- SINGLE OUTPUT UP TO 4A

- 4:1 ULTRA WIDE INPUT VOLTAGE RANGE

- INPUT TO OUTPUT ISOLATION (BASIC INSULATION)

- INDUSTRY STANDARD PIN-OUT FEC15 SERIES COMPATIBLE

- SURFACE-MOUNT OR THROUGH-HOLE

- COST EFFIECIENT OPEN FRAME DESIGN

- SMALL SIZE AND LOW PROFILE：1.10 x 0.94 x 0.335 Inch

- FIXED SWITCHING FREQUENCY

- RoHS DIRECTIVE COMPLIANT

VER:01    1  /  2

  

LED15W single output DC/DC converters provide up to 15 watts of output power in an industry standard package and footprint. These units are specifically designed to meet the power needs of low profile. All models feature with 4:1 ultra wide input voltage of  9-36 VDC and 18-75 VDC ,comprehensively protected against over-current, over-voltage and input under-voltage protection conditions, and trimmable output voltage.

The LED15W converters are especially suited to Network, Data processing, Wireless and Enterprise equipment and microprocessor, intermediate bus voltage power application.

UL

TUV

CB

CE MARK

(Pending)

| OUTPUT SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output power |  |  |  |  |  |  |  |  |  |  |  | 15 Watts |  |
| Voltage accuracy |  |  | Full load and nominal Vin |  |  |  |  |  |  |  |  | ± 1% |  |
| Voltage adjustability (Note 1) |  |  |  |  |  |  |  |  |  |  |  | ±10% |  |
| Line regulation |  |  | LL to HL at Full Load |  |  |  |  |  |  |  |  | ± 0.2% |  |
| Load regulation |  |  | No Load to Full Load |  |  |  |  |  |  |  |  | ± 0.2% |  |
| Ripple and noise |  |  | 20MHz bandwidth (Measured with a 1uF M/C and a 10uF T/C) |  |  |  |  |  |  |  |  | See table |  |
| Temperature coefficient |  |  |  |  |  |  | ±0.02% / ºC, max |  |  |  |  |  |  |
| Transient response recovery time |  |  |  |  | 25% load step change |  |  |  |  | 250uS |  |  |  |
| Over voltage protection |  |  | 3.3V output 5V output 12V output 15V output |  |  |  |  |  |  |  |  | 3.7V-5.4V 5.6V-7.0V 13.8V-17.5V 16.8V-20.5V |  |
| Over load protection |  |  | % of FL at nominal input |  |  |  |  |  |  |  |  | 150% max |  |
| Short circuit protection |  |  | Hiccup, automatics recovery |  |  |  |  |  |  |  |  |  |  |
| INPUT SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Input voltage range |  |  |  | 24V nominal input 48V nominal input |  |  |  |  |  |  | 9 – 36VDC 18 – 75VDC |  |  |
| Under voltage lockout |  |  |  | 24V input 48V input |  |  |  | DC-DC ON DC-DC OFF DC-DC ON DC-DC OFF |  |  | 9VDC 8VDC 18VDC 16VDC |  |  |
| Input voltage variation |  | dv/dt |  | 5V/ms,max (Complies with ETS300 132 part 4.4) |  |  |  |  |  |  |  |  |  |
| Input surge voltage 100mS max |  |  |  | 24V input 48V input |  |  |  |  |  |  | 50VDC 100VDC |  |  |
| Input reflected ripple current |  |  |  | Nominal Vin and full load |  |  |  |  |  |  | 30mAp-p |  |  |
| Start up time | Nominal Vin and constant resistive load |  |  |  |  |  |  | Power up RemoteON/OFF |  |  |  |  | 30ms, Max. 30ms, Max. |
| Remote ON/OFF (Note 2) (Positive logic) (Negative logic) Input current of Remote control pin Remote off state input current |  |  |  |  |  | DC-DC ON DC-DC OFF DC-DC ON DC-DC OFF Nominal input |  |  | Open or 3 V < Vr < 12V Short or 0V < Vr < 1.2V Short or 0V < Vr < 1.2V Open or 3V < Vr < 12V -0.5mA~1.0mA TBD |  |  |  |  |

TECHNICAL SPECIFICATION All specifications are typical at nominal input, full load and 25ºC otherwise noted

| GENERAL SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Efficiency |  |  |  | See table |  |  |  |  |  |  |  |  |
| Isolation voltage |  | Input to Output(Basic Insulation) |  |  |  |  |  |  |  |  |  | 2250VDC, min |
| Isolation resistance |  |  | 109ohms, min |  |  |  |  |  |  |  |  |  |
| Isolation capacitance |  |  | 1000pF, max |  |  |  |  |  |  |  |  |  |
| Switching frequency |  |  | 3.3V,5V 12V,15V |  |  |  |  |  | 300KHz 400KHz |  |  |  |
| Safety standard pending |  |  | IEC60950-1, UL60950-1, EN60950-1 |  |  |  |  |  |  |  |  |  |
| Dimensions |  |  | 1.10 X 0.94 X 0.335 Inch (27.94 X 23.88 X 8.5 mm) |  |  |  |  |  |  |  |  |  |
| Weight |  |  | 10.5g(0.36oz) |  |  |  |  |  |  |  |  |  |
| MTBF (Note 3) |  |  | BELLCORE TR-NWT-000332 MIL-STD-217F |  |  |  |  |  |  | TBD TBD |  |  |
| ENVIRONMENTAL SPECIFICATIONS |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating ambient temperature |  |  |  |  |  | TBD |  |  |  |  |  |  |
| Storage temperature range |  |  |  |  |  |  |  | -55ºC ~ +125ºC |  |  |  |  |
| Thermal shock |  |  |  |  |  |  |  |  |  |  | MIL-STD-810D |  |
| Vibration | 10-55HZ,2G,3minutes period,30 minutes along X,Y and Z |  |  |  |  |  |  |  |  |  |  |  |
| Relative humidity | 5% to 95% RH |  |  |  |  |  |  |  |  |  |  |  |
| EMC CHARACTERISTICS (Note4) |  |  |  |  |  |  |  |  |  |  |  |  |
| Conducted emissions |  |  |  |  | EN55022 |  | Class A |  |  |  |  |  |
| Radiated emissions |  |  |  |  | EN55022 |  | Class A |  |  |  |  |  |
| ESD |  |  |  |  | EN61000-4-2 |  | Perf. Criteria B |  |  |  |  |  |
| Radiated immunity |  |  |  |  | EN61000-4-3 |  | Perf. Criteria A |  |  |  |  |  |
| Fast transient |  |  |  |  | EN61000-4-4 |  | Perf. Criteria B |  |  |  |  |  |
| Surge |  |  |  |  | EN61000-4-5 |  | Perf. Criteria B |  |  |  |  |  |
| Conducted immunity |  |  |  |  | EN61000-4-6 |  | Perf. Criteria A |  |  |  |  |  |

VER:01     2  /  2

| Model Number | Input Range | Output Voltage | OutputCurrent |  | Output Ripple&Noise | Input Current |  | Eff (7) (%) | Capacitor (8) Load max |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min.Load | Full Load |  | No Load (5) | Full Load (6) |  |  |
| LED15-24S3P3W | 9 - 36 VDC | 3.3 VDC | 0mA | 4000mA | 75mVp-p | 50mA | 688mA | 84 | 1000uF |
| LED15-24S05W | 9 - 36 VDC | 5 VDC | 0mA | 3000mA | 75mVp-p | 50mA | 782mA | 84 | 1000uF |
| LED15-24S12W | 9 - 36 VDC | 12 VDC | 0mA | 1300mA | 100mVp-p | 20mA | 803mA | 85 | 330uF |
| LED15-24S15W | 9 - 36 VDC | 15 VDC | 0mA | 1000mA | 100mVp-p | 20mA | 772mA | 85 | 220uF |
| LED15-48S3P3W | 18 - 75 VDC | 3.3 VDC | 0mA | 4000mA | 75mVp-p | 40mA | 336mA | 86 | 1000uF |
| LED15-48S05W | 18 - 75 VDC | 5 VDC | 0mA | 3000mA | 75mVp-p | 40mA | 382mA | 86 | 1000uF |
| LED15-48S12W | 18 - 75 VDC | 12 VDC | 0mA | 1300mA | 100mVp-p | 15mA | 392mA | 87 | 330uF |
| LED15-48S15W | 18 - 75 VDC | 15 VDC | 0mA | 1000mA | 100mVp-p | 15mA | 377mA | 87 | 220uF |

Note

- Trimming allows the user to increase or decrease the output voltage set point of the module. This is accomplished by connecting an external resistor between the TRIM pin and either the +VOUT pin or the -VOUT pin.

- The pin voltage is referenced to -Vin.The order number please see product standard table.

- BELLCORE TR-NWT-000332. Case 1: 50% Stress, Temperature at 40ºC. (Ground fixed and controlled environment)

MIL-STD-217F Notice2 @Ta=25 ºC, Full load(Ground, Benign, controlled environment)

- The LED15W series required external filter to meets EN55022 class A(TBD).

- Typical value at nominal input voltage.

- Maximum value at nominal input voltage and full load.

- Typical value at nominal input voltage and full load

- Test by minimum Vin and constant resistive load.

EXTERNAL OUTPUT TRIMMING

6

5

4

TRIM

UP

TRIM

DOWN

RT1or

RT2

Output can be externally trimmed by using the method shown below.

PIN CONNECTION

PIN

LED15W SERIES

1

+ INPUT

2

- INPUT

3

ON/OFF

4

+VOUT

5

TRIM

6

-VOUT

PAD LAYOUT 6 PADS Φ2.8mm

50

60

70

80

90

105

-40

-25

0

-10

25

50

75

100

AMBIENT TEMPERATURE (ºC)

OUTPUT POWER (%)

LED15-48S05W Derating Curve

48Vin

18Vin

75Vin

Natural convection

TBD

200LFM

50

60

70

80

90

105

-40

-25

0

-10

25

50

75

100

AMBIENT TEMPERATURE (ºC)

OUTPUT POWER (%)

LED15-48S05W Derating Curve

500LFM

400LFM

300LFM

TBD

1.All dimensions in inches(mm)

2.Tolerance : x.xx±0.02(x.x±0.5)

x.xxx±0.010(x.xx±0.25)

3.Pin pitch tolerance ±0.014(0.35)

PRODUCT STANDARD TABLE

| Option | Suffix |
| --- | --- |
| Negative remote ON/OFF with DIP |  |
| Negative remote ON/OFF with SMT | -A |
| Positive remote ON/OFF with DIP | -B |
| Positive remote ON/OFF with SMT | -C |
| DIP type without ON/OFF pin | -D |
| SMT type without ON/OFF pin | -E |
| DIP type,negative remote ON/OFF without TRIM pin | -F |
| SMT type,negative remote ON/OFF without TRIM pin | -G |
| DIP type without ON/OFF&TRIM pin | -H |
| SMT type without ON/OFF&TRIM pin | -I |
| DIP type,positive remote ON/OFF without TRIM pin | -J |
| SMT type,positive remote ON/OFF without TRIM pin | -K |

Nature Convection